Introduction to Computer Architecture - Answers 2013

All questions are compulsory, questions are weighted equally.

Answer to Question 1

Qa,b,d are easy questions testing basic knowledge & understanding.C tests ability to optimise code (i) is bookwork.

1.

a)

(i)	13631489     (223*0b1.101 000 000 000 0001)	2

(ii)	13631490    (223*0b1.101 000 000 000 0010)	2

(iii)	As (i) but exp is one larger => X2.

Note that the exp=24, so difference between successive IEEE-754 numbers in this range is 2. Half of this (1) is the max deviation from the nominal value:

27262977 – 27262979 (13631489*2 ±1)	2

# Common mistakes were to get exponent wrong, or to give an approximate answer (calculator will not have enough precision unless used carefully)

[6]

b)

(i)	0x34D25	1

(ii)	0x03	1

(iii)	invalid, will cause memory abort exception	1

Many did not identify that (iii) was invalid, and that it would result in an exception.

[3]

c)

(i)	ADD R1, R2, R2, lsl #6	3

(ii)	RSC R3, R3, R4, lsl #1 ; works because C=0 	3

Many took 2 or mor instructions to implement (ii)

[6]

d)

T = 100MHz/(1+3*0.3*0.25) = 81.6MHz (or 0.816 instructions/cycle)	3

Typical code will contain many LDR/STR instructions which also cause pipeline

stalls, so decreasing throughput.	2

[5]

Answer to Question 2

This question tests ability to understand and analyse operation of ARM assembly code in detail. It requires accuracy and comprehensive understanding of the instructions, but is straightforward.

- tests understanding of two’s complement arithmetic.

- test understanding of shift and rotate instructions.

- tests understanding  of LDR/LDRB instructions.

# Marks are awarded 1 per answer not flagged n/a. Note that condition codes CV and NZ are counted as separate answers.

Mistakes here were too varied to categorise. Worth noting that many lost marks with incorrect condition codes.

| Location (word) | Value |
| --- | --- |
| 0x100 | 0x04030201 |
| 0x104 | 0x08070605 |
| 0x108 | 0x0F0C0D0A |
| >0x108 | 0 |

Figure 2.1. Memory locations

MOV R4, #1

MOV R5, #-2

ADDS R0, R4, R5

SBCS R1, R5, R4

RSB R2, R4, #80000000

MVNS R3, #0

(a)

MOV 	R1, #10

MOV 	R2, #3

MOV 	R3, #0

MOVS R2, R2, ror #1

ADDMI R3, R3, R1

MOVS R2, R2, ror #1

ADDMI R3, R3, R1, lsl #1

MOVS R2, R2, ror #1

ADDMI, R3, R3, R1, lsl #2

(b)

MOV  R4, #0x104

ANDS R3, R4, #7

LDRB  R0, [R4, #3]

LDRB  R1, [R4, #-1]

LDR	 R2, [R4, R3, lsl #1]

(c)

Figure 2.2. Code fragments

|  | R0 | R1 | R2 | R3 | R4 | NZ CV | Time |  | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (a) | -1 | -4 | 0x4C4B3FF 79999999 | 0xFFFFFFFF -1 | 1 | 10 10 | n/a |  | 7 |
| (b) | n/a | 10 | 0x60000000 1610612736 | 30 | n/a | 00 00 | 9 |  | 6 |
| (c) | 8 | 4 | 0 | 4 | 0x104 260 | 00 n/a | 14 |  | 7 |

Figure 2.3. Template for answers

Answer to Question 3

This question tests understanding of subroutines, and operation of LDM/STM instructions, as well as how to optimise sequential access to memory.

a)

mem32[a] := mem32[b]	2

mem32[a+4] := mem32[b+4]	1

mem32[a+8] := mem32[b+8]	1

execution time = 4 (STM) + 6*4 (LDR/STR) + 4(LDM) + 4  (MOV) + 4 (BL) = 40	2

[allow 1/2 for time if working shows STM, LDR/STR, and LDM time is all correct]

This question was well answered by those who worked out what the memory transfer was. Some gave incorrect answers in involving registers.

[6]

b)

1:

mem32[c] := R2

R13 := R13+4  (push R2)

8/9:

R2 := mem32[c]

R13 := R13 – 4  (pop R2)

PC := R14

R2 is pushed onto an empty ascending stack with SP R13, and then popped back off it at end. 		2

Finally a branch is made to the address stored in R14 (which will be the return address written by a BL instruction).		1

R2 must be saved on stack because it is used as working register by subroutine.		1

[4]

Many missed the reasons for the operations

c)

LDMIA R1, {R2,R3,R4}		2

STMIA R0, {R2,R3, R4}		2

24 -12 = 12 cycles		1

Some got the direction of mem transfers wrong here.

Common mistake was to increment the base registers R1,R0, that must not happen

[5]

d)

1	MOVE		STMEA	R13!, {R2,R3,R4}

2			LDMIA 	R1, {R2,R3,R4}

3			STMIA 	R0, {R2,R3,R4}

4			LDMEA 	R13!, {R2,R3,R4}

5			MOV 		PC, R14

4

Total time saved = 12 – 4 (longer STMED/LDMED) = 8 cycles		1

Most found this question quite easy.

[5]