Special instructions for invigilators

The sheet Exam Notes should be distributed with the Examination Paper.

Special instructions for students

The prefix 0x, or suffix (16), introduces a hexadecimal number, e.g: 0x1C0, 1C0(16).

Unless otherwise specified, negative numbers are represented in two's complement.

Unless otherwise specified, machine addressing is little-endian.

The sheet Exam Notes, as published on the course web pages, is provided and contains reference material.

Answer ALL the questions.

The Questions

1.

a)

(i) 	What is the decimal value represented by 0x4B500001 in IEEE-754?

(ii) 	What is the decimal value represented by 0x4B500002 in IEEE-754?

(iii) 	A real number x has nearest IEEE-754 representation 0x4BD00001. What in decimal are the highest and lowest possible values of x?

[6]

b)	32 bit little-endian ARM memory is as shown in Figure 1.1. Give the value of each of the expressions below in hexadecimal, or if the expression is not valid state what action an ARM CPU would take on a load instruction referencing the expression.

(i)	mem32[0x104]

(ii)	mem8[0x101]

(iii)	mem32[0x102]

[3]

c)	Write efficient separate ARM code fragments which will perform the specified operations, ignoring overflow. You may assume that C = 0 on entry to the fragment:

(i)	R1 := 65*R2

(ii)	R3 := 2*R4 - R3 - 1

[6]

d)	A 100MHz ARM7 CPU executes branch instructions 30% of the time, and 75% of branch instructions are condition false executed. Assuming no instruction causes pipeline stalls other than branches what is the CPU throughput? Give one specific reason why you would expect typical CPU throughput with these branch instruction execution statistics to be less than this.

[5]

| Word address in hexadecimal | Value in unsigned decimal |
| --- | --- |
| 0x100 | 1003 |
| 0x104 | 216357 |

Figure 1.1

2.

Each code fragment (a) - (c) below executes with all condition codes and registers initially 0, and memory locations as in Figure 2.1. State as required the values in R0-R4 and the condition codes after execution of the code fragment. State the execution time in cycles of the instructions. Write your answers using as a template a copy of the table in Figure 2.3, deleting the example row labelled (x) which indicates the required format of your answer. Each answer must be written with register values in signed decimal and condition codes in binary. The example row (x) shows this. Where an answer is labelled n/a you need not give it.

a)	Code as in Figure 2.2a.

[7]

b)	Code as in Figure 2.2b.

[6]

c)	Code as in Figure 2.2c.

[7]

| Location (word) | Value |
| --- | --- |
| 0x100 | 0x04030201 |
| 0x104 | 0x08070605 |
| 0x108 | 0x0F0C0D0A |
| >0x108 | 0 |

Figure 2.1. Memory locations

MOV R4, #1

MOV R5, #-2

ADDS R0, R4, R5

SBCS R1, R5, R4

RSB R2, R4, #80000000

MVNS R3, #0

(a)

MOV 	R1, #10

MOV 	R2, #3

MOV 	R3, #0

MOVS R2, R2, ror #1

ADDMI R3, R3, R1

MOVS R2, R2, ror #1

ADDMI R3, R3, R1, lsl #1

MOVS R2, R2, ror #1

ADDMI R3, R3, R1, lsl #2

(b)

MOV  R4, #0x104

ANDS R3, R4, #7

LDRB  R0, [R4, #3]

LDRB  R1, [R4, #-1]

LDR	 R2, [R4, R3, lsl #1]

(c)

Figure 2.2. Code fragments

|  | R0 | R1 | R2 | R3 | R4 | NZCV | Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (x) | 0 | 222 | -33 | 4 |  | 0110 | 3 |
| (a) |  |  |  |  |  |  | n/a |
| (b) | n/a |  |  |  | n/a |  |  |
| (c) |  |  |  |  |  |  |  |

Figure 2.3. Template for answers

3.

a)	Figure 3.1 shows a listing of a subroutine with inputs R0, R1 which changes memory locations. Suppose that R0, R1, R13 have values a, b, c when the subroutine is called. State precisely what changes to memory locations, other than stack, are made by the subroutine, and what is the total execution time of the subroutine including the branch and link instruction that calls it.

[6]

b)	If R13 = c on entry to the subroutine, state what changes to memory locations and registers are made by instructions 1, 8 and 9 of Figure 3.1, and explain why these are necessary.

[4]

c)	Show how instructions 2-7 of Figure 3.1 can be implemented more efficiently using LDM/STM instructions, and state the number of execution cycles saved by this optimisation.

[5]

d)	Using your optimised code from part c, rewrite the subroutine, with appropriate changes to instructions 1 and 8. What, overall, is the saving in execution time of the optimised subroutine compared with the code in Figure 3.1?

[5]

1	MOVE		STMEA	R13!, {R2}

2			LDR 		R2, [R1]

3			STR 		R2, [R0]

4			LDR 		R2, [R1,#4]

5			STR 		R2, [R0,#4]

6			LDR 		R2, [R1,#8]

7			STR 		R2, [R0,#8]

8			LDMEA 	R13!, {R2}

9			MOV 		PC, R14

Figure 3.1.

Introduction to Computer Architecture - Answers 2013

All questions are compulsory, questions are weighted equally.

Answer to Question 1

Qa,b,d are easy questions testing basic knowledge & understanding.C tests ability to optimise code (i) is bookwork.

1.

a)

(i)	13631489     (223*0b1.101 000 000 000 0001)	2

(ii)	13631490    (223*0b1.101 000 000 000 0010)	2

(iii)	As (i) but exp is one larger => X2.

Note that the exp=24, so difference between successive IEEE-754 numbers in this range is 2. Half of this (1) is the max deviation from the nominal value:

27262977 – 27262979 (13631489*2 ±1)	2

# Common mistakes were to get exponent wrong, or to give an approximate answer (calculator will not have enough precision unless used carefully)

[6]

b)

(i)	0x34D25	1

(ii)	0x03	1

(iii)	invalid, will cause memory abort exception	1

Many did not identify that (iii) was invalid, and that it would result in an exception.

[3]

c)

(i)	ADD R1, R2, R2, lsl #6	3

(ii)	RSC R3, R3, R4, lsl #1 ; works because C=0 	3

Many took 2 or mor instructions to implement (ii)

[6]

d)

T = 100MHz/(1+3*0.3*0.25) = 81.6MHz (or 0.816 instructions/cycle)	3

Typical code will contain many LDR/STR instructions which also cause pipeline

stalls, so decreasing throughput.	2

[5]

Answer to Question 2

This question tests ability to understand and analyse operation of ARM assembly code in detail. It requires accuracy and comprehensive understanding of the instructions, but is straightforward.

- tests understanding of two’s complement arithmetic.

- test understanding of shift and rotate instructions.

- tests understanding  of LDR/LDRB instructions.

# Marks are awarded 1 per answer not flagged n/a. Note that condition codes CV and NZ are counted as separate answers.

Mistakes here were too varied to categorise. Worth noting that many lost marks with incorrect condition codes.

| Location (word) | Value |
| --- | --- |
| 0x100 | 0x04030201 |
| 0x104 | 0x08070605 |
| 0x108 | 0x0F0C0D0A |
| >0x108 | 0 |

Figure 2.1. Memory locations

MOV R4, #1

MOV R5, #-2

ADDS R0, R4, R5

SBCS R1, R5, R4

RSB R2, R4, #80000000

MVNS R3, #0

(a)

MOV 	R1, #10

MOV 	R2, #3

MOV 	R3, #0

MOVS R2, R2, ror #1

ADDMI R3, R3, R1

MOVS R2, R2, ror #1

ADDMI R3, R3, R1, lsl #1

MOVS R2, R2, ror #1

ADDMI, R3, R3, R1, lsl #2

(b)

MOV  R4, #0x104

ANDS R3, R4, #7

LDRB  R0, [R4, #3]

LDRB  R1, [R4, #-1]

LDR	 R2, [R4, R3, lsl #1]

(c)

Figure 2.2. Code fragments

|  | R0 | R1 | R2 | R3 | R4 | NZ CV | Time |  | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (a) | -1 | -4 | 0x4C4B3FF 79999999 | 0xFFFFFFFF -1 | 1 | 10 10 | n/a |  | 7 |
| (b) | n/a | 10 | 0x60000000 1610612736 | 30 | n/a | 00 00 | 9 |  | 6 |
| (c) | 8 | 4 | 0 | 4 | 0x104 260 | 00 n/a | 14 |  | 7 |

Figure 2.3. Template for answers

Answer to Question 3

This question tests understanding of subroutines, and operation of LDM/STM instructions, as well as how to optimise sequential access to memory.

a)

mem32[a] := mem32[b]	2

mem32[a+4] := mem32[b+4]	1

mem32[a+8] := mem32[b+8]	1

execution time = 4 (STM) + 6*4 (LDR/STR) + 4(LDM) + 4  (MOV) + 4 (BL) = 40	2

[allow 1/2 for time if working shows STM, LDR/STR, and LDM time is all correct]

This question was well answered by those who worked out what the memory transfer was. Some gave incorrect answers in involving registers.

[6]

b)

1:

mem32[c] := R2

R13 := R13+4  (push R2)

8/9:

R2 := mem32[c]

R13 := R13 – 4  (pop R2)

PC := R14

R2 is pushed onto an empty ascending stack with SP R13, and then popped back off it at end. 		2

Finally a branch is made to the address stored in R14 (which will be the return address written by a BL instruction).		1

R2 must be saved on stack because it is used as working register by subroutine.		1

[4]

Many missed the reasons for the operations

c)

LDMIA R1, {R2,R3,R4}		2

STMIA R0, {R2,R3, R4}		2

24 -12 = 12 cycles		1

Some got the direction of mem transfers wrong here.

Common mistake was to increment the base registers R1,R0, that must not happen

[5]

d)

1	MOVE		STMEA	R13!, {R2,R3,R4}

2			LDMIA 	R1, {R2,R3,R4}

3			STMIA 	R0, {R2,R3,R4}

4			LDMEA 	R13!, {R2,R3,R4}

5			MOV 		PC, R14

4

Total time saved = 12 – 4 (longer STMED/LDMED) = 8 cycles		1

Most found this question quite easy.

[5]